# CS 4323 — Course Project Data Review (Revised)

Specific Emitter Identification under Maritime Channel Distortion

One sentence description: a multiclass classification model over K emitters, trained on I/Q samples, with Bayesian uncertainty quantification evaluated under distribution shift.

Revision note. This version responds to instructor feedback on the first submission (that signal datasets of this type are commonly converted to spectrograms via short-time Fourier transform before applying a CNN). Section 4 now treats input representation as an experimental variable rather than a fixed choice. Dataset figures have also been replaced with exact counts verified against the downloaded file.

## 1. Dataset

About the source data: WiSig: A Large-Scale WiFi Signal Dataset for Receiver and Channel Agnostic RF Fingerprinting (Hanna, Karunaratne, and Cabric, IEEE Access, vol. 10, pp. 22808–22818, 2022). Publicly released by the UCLA CORES Lab under a BSD-3-Clause license; captured on the ORBIT testbed. The project uses the pre-packaged ManyTx subset (2.5 GB compressed, 4.18 GB expanded).

What ManyTx contains. ManyTx contains IEEE 802.11a/g preamble captures collected from 150 physical WiFi transmitters, recorded by 18 physical USRP receivers, across 4 capture days (1, 8, 15, and 23 March 2021). Each transmitter–receiver–day combination holds up to 50 signals. Inspection of the downloaded file gives an exact total of 511,515 non-equalized signals (509,128 equalized). Coverage is highly even across receivers (27,887–29,114 signals each) and across days (127,276–128,478 each), so no receiver or day is data-starved and any one can serve as a held-out condition.

Per-transmitter coverage is less uniform. The theoretical maximum is 3,600 signals per transmitter (18 receivers × 4 days × 50); the observed range is 1,556 to 3,600, with a median of 3,578. 71 of the 150 transmitters have complete coverage at 3,600 signals, and the planned experiments draw their emitter cohort from that balanced subset so that every class is equally represented across all receivers and days.

The dataset ships in both non-equalized and equalized forms; this project uses the non-equalized version, because channel equalization removes propagation effects that a passive intercept receiver would not be able to undo, and because the project deliberately adds its own channel distortion downstream.

Because the transmitters are physical devices, the hardware impairments that constitute the RF fingerprint (I/Q imbalance, power-amplifier nonlinearity, oscillator phase noise, clock jitter) are genuine rather than simulated, as are the receiver-side imperfections.

Preprocessing added by this project, for the purpose of measuring the reliability of the model’s ability to identify these signals given real-world distortions. Captures are passed through a maritime channel-augmentation stage that applies two-ray multipath, Doppler scaled to the carrier frequency, evaporation ducting, and additive noise spanning −5 dB to 0 dB SNR, at three severity tiers (Controlled, Degraded, Dynamic). Signals are power-normalized per burst. Arrays are stored on disk as float64 and are cast to float32 on load, halving memory with no meaningful loss of precision for this task. The processed dataset is stored in the Signal Metadata Format (SigMF) so that every example records its source transmitter, source receiver, capture day, and augmentation parameters.

Rather than a random shuffle, splits are constructed to create deliberate distribution shift: one severity tier, one physical receiver, and one capture day are withheld entirely from training and appear only at test time. This allows generalization to unseen environmental and receiver conditions to be measured directly.

## 2. Inputs and Outputs

Input (x). One captured 802.11 preamble of 256 complex baseband samples, stored in the source file as a real-valued array of shape (256, 2) with in-phase and quadrature components as separate channels. Two representations are derived from this array and compared experimentally; both are described in Section 4.

Output (y). A single integer class label in {0, …, K−1} identifying which physical transmitter produced the burst, where the index maps to an ORBIT node ID through a frozen lookup table stored with the dataset. Ground truth is known exactly because the transmitting node was recorded at capture time.

Why this is a multiclass problem. The K transmitters are mutually exclusive options, and exactly one of them produced any given burst. The task is therefore multiclass rather than binary, which would admit only two outcomes, and rather than multi-label, which would allow several transmitters to be assigned to the same capture. This structure is what motivates the modeling choices that follow: a categorical predictive distribution formed by a softmax over K logits, categorical cross-entropy as the training loss, and per-class accuracy as the reported metric.

Number of classes (K). The full subset supports K = 150, of which 71 transmitters have complete coverage. Class count is treated as an experimental variable, with runs at approximately K = 10, 50, and 71 drawn from the balanced subset, to observe how both accuracy and calibration degrade as the emitter population grows. A K = 150 run is possible but requires class weighting to compensate for the uneven coverage noted above.

## 3. Prediction Distribution

The predictive distribution is categorical over the K transmitters, produced by a softmax over the network’s K output logits:

p(y = k | x, θ) = exp(zₖ) / Σⱼ exp(zⱼ)

Training therefore minimizes the multiclass classification loss (categorical cross-entropy), which is equivalent to maximum likelihood estimation of this categorical model. Adding a Gaussian prior over the weights, equivalently L2 regularization implemented as weight decay, gives MAP estimation.

For the approximate Bayesian methods, the categorical likelihood is unchanged; what changes is that the weights are drawn from an approximate posterior rather than fixed. The reported prediction is the posterior predictive distribution, obtained by Bayesian model averaging over S sampled weight settings:

p(y | x, D) ≈ (1/S) Σₛ p(y | x, θ⁽ˢ⁾)

The spread across those S samples supplies the uncertainty estimate this project evaluates for calibration. That spread is further decomposed into an aleatoric component, reflecting noise inherent to the signal and irreducible with more data, and an epistemic component, reflecting uncertainty in the model’s own parameters. Epistemic uncertainty is the quantity of interest, since it is what should rise on conditions absent from training. Calibration is reported as a reliability diagram (predicted confidence against observed accuracy, where a well-calibrated model traces the diagonal) together with expected calibration error, computed separately for each test split.

## 4. Input Representation and Model Architecture

Input representation is treated as an experimental variable. Two front ends are compared, with the same inference methods applied to each:

- Raw I/Q, presented as a 2 × 256 tensor and consumed by a one-dimensional CNN. This is the default in much of the SEI literature.

- Spectrogram, obtained by short-time Fourier transform of the burst and consumed by a two-dimensional CNN, with magnitude and phase supplied as separate channels.

The second representation is motivated by this project’s emphasis on channel robustness. A propagation channel acts on the transmitted signal as a convolution in time, which entangles the hardware fingerprint with channel effects in the raw samples. In the frequency domain that convolution becomes a product, and in log-magnitude an additive offset, which a network can more readily discount. Reported comparisons in the RF fingerprinting literature find spectrogram inputs competitive with, and often superior to, raw I/Q for channel-robust identification.

Phase is retained deliberately. A magnitude-only spectrogram discards phase information, yet I/Q imbalance and power-amplifier AM/PM distortion are partly phase effects, so magnitude and phase are supplied as two channels rather than magnitude alone. Because each capture is only 256 samples long, STFT window length and overlap materially affect the resolution of the resulting spectrogram; these parameters will be tuned and the chosen values reported.

Architecture: a neural network, not a linear model. The dominant fingerprint mechanisms are nonlinear (power-amplifier AM/AM and AM/PM distortion, multiplicative phase noise), so the mapping from a received signal to emitter identity cannot be captured by a linear decision boundary under either representation.

Planned structure: three to four convolutional blocks (Conv → ReLU → pooling), applied one-dimensionally along the time axis for the raw I/Q front end and two-dimensionally over time and frequency for the spectrogram front end, with dropout layers between blocks, followed by a small fully connected layer and a final linear layer producing K logits. The network is kept deliberately small, on the order of a few hundred thousand parameters, which is sufficient for short preamble segments, keeps the Laplace computation tractable, and should train comfortably on a single GPU. Dropout is included from the outset because it serves as the variational distribution for the Monte Carlo dropout method later in the project.

The architecture is held fixed across all four inference methods (MLE, MAP, Laplace approximation with a diagonal empirical Fisher approximation to the Hessian, and Monte Carlo dropout), so that any difference in accuracy or calibration is attributable to the inference method rather than to the model. Comparing the two front ends under this fixed protocol allows a question the reviewed literature does not address: whether a time-frequency representation improves not only accuracy under channel shift, but calibration as well.